The paper examines whether the Posit number format offers a practical hardware alternative to IEEE 754 float. Although Posit is often considered more precise and bit-efficient, the study aimed to measure its actual hardware cost in terms of area, delay, and resource usage. Using a dedicated C++ hardware library (MArTo), the authors implemented identical arithmetic units—addition, multiplication, and accumulation—for both formats, and tested them on FPGAs using Vivado HLS and Quartus. Real synthesis was performed on Posit8 to Posit64, with comparisons to FP16, FP32, and FP64 using the sfpy library.

The results showed that Posit consistently required more resources and longer delays than float for equivalent bit widths. For example, 32-bit addition with Posit used 719 LUTs and 7 cycles, compared to 435 LUTs and 6 cycles for IEEE float. In a benchmark involving 1000 multiplications and additions, Posit32 with Quire achieved high accuracy but at a high cost, while standard float was faster but less precise.

The conclusion is that Posit is not cheaper than IEEE float, and often more complex to implement. However, it may be worth the cost in workloads where numerical precision and stability matter more than hardware efficiency. Its use is best considered selectively, depending on the application.