תמלול מצגת

שקף 2

The goal of our project was to research and explore the open-source RISC-V architecture, and to design and implement it as a soft-core on an FPGA.

We used several implementation methods to determine which one has the best performance and reliability.

The project includes RV32IM soft-core running on an Artix-7 FPGA, also six different assembly programs that run on it.

שקף 3

RISC-V (reduced instruction set computer) is a modern open-source, and modular instruction set architecture.
It was developed at the Parallel Computing lab at the University of California, to provide a free and scalable ISA that can be used in a wide range of applications.

The RISC-V ISA operates according to the RISC philosophy, meaning it uses a small set of simple instructions, each focused on executing one operation.

In RISC-V there are 6 different formats:

- R-type – for arithmetic and logical operations between two registers.

- I-type – for arithmetic and logical/the same operations with an immediate, loads, jumps and system calls.

- S-type – for store operations.

- B-type – for conditional jumps.

- U-type – for upper immediate operations.

- J-type – for jumps.

ABI (application binary interface) defines how the binary code communicates with the operating system. It gives us:

- The registers names and their role.

- Which registers are saved by the callee or caller.

- Which arguments are supposed to be passed to a function and restored on each stage.

As we mentioned, RISC-V is a modular architecture which means you can implement what extensions you want to use according to your system needs.

For example we have the

- M extension for multiplication and division operations.

- C extension for compressed instructions.

- F/D – for single and double precision floating point.

There are several architectural approaches for implementing a RISC-V processor, such as single-cycle, multi-cycle, and 3/5/7 stage pipeline and superscalar.
In this project, we chose to implement the first three

שקף 4

RISC-V processors are built from several main components —

the PC register , instruction memory, register file, extend unit, ALU, and data memory.

Program Counter (PC): Holds the address of the next instruction  that should be executed.

Instruction Memory : Stores the program instructions

Register File: A collection of registers used for temporary data storage during instruction execution.

Extend Unit: Expands immediate values to the required bit-width.

ALU (Arithmetic Logic Unit): who execute  arithmetic and logical operations such as add sub or and more.

Data Memory: Used for reading and writing data during program execution.

The way these components are connected determines the processor’s architecture.

So, as you can see, in the left side of the screen, we have the pipeline and the single cycle structures. There are a few differences between in the way each one is working.

In the pipelined processor, registers are placed between the stages, allowing instructions to be executed in parallel — each instruction continue for a few stages and multiple cycles.

In contrast, in the single cycle processor, every instruction is completed within one long clock cycle, which makes it slower compared to the shorter cycles of the pipelined design.

Each design is divided into two sections

the Datapath and the control unit.
The Datapath contains all the components that are responsible for executing the instruction, while the control unit ensures that only the required components in the datapath are activated at the right time for correct execution.

For the multi-cycle approach, we found that the best way to implement the control unit was by using a finite state machine(moore – there are dependency only for the current state and the changes occur only when we move between stages) that moves between states, each defining the control signals needed for that cycle.

שקף 5

Ok, in the beginning, when we decided to work with FPGA, there is a few design flow we had to follow by.

The first stage is design. We planned and described our processor architecture. Then and based on that we implemented all the modules in System Verilog and defined all the inputs and outputs of the system. By the end of this stage, we had a RTL design (Register Transfer Level) of our Processor.

The second stage is simulation. Here, we verified that our design worked correctly. We wrote six programs to test all of the instructions and processor’s behavior and Each program ran on the processor through a dedicated testbench. It took us a a lot of time to verify our self. So, to speed up the verification time, we created a HASH code that immediately indicated us if the processor produced the correct results.

The  next stage is Synthesis. In this stage we used the built in tools in Vivado to convert our SV code into a hardware netlist.

Then we implement - we used Vivado’s tools to physically map and connect the synthesized design inside the FPGA. By this stage we already had a  routed design and diagram showing exactly how it would look on the FPGA.

Finally, we generated the bitstream and programmed it into the FPGA. After the bitstream was programmed, we could see the processor running and test it live on the FPGA.

Yuval

In the end of the project, we had in our hand a functioning RV32IM Processor which was program in system Verilog.

We implemented the processor various ways including: Single cycle, Multi Cycle and Pipelined. This choice allowed us to gain full understanding of RISC-V and Computer Architecture.

We also wrote 6 different assembly code which we later translated to Machine code. These files were used in the simulation to check the Processor and were programed into the BRAM of the FPGA to see that it works there as well.

שקף 7

We had few challenges throughout our project.

When working on the pipelined processor we needed to design a hazard unit that will solve several problems that may occur while it runs.

Some of the problems are data  and control hazards.

Data hazards may occur when data that is needed in a following instruction is yet to be updated in the register file. We solved this problem by forwarding the data from the first instruction to the instruction that needed the data before it written back into the register file.

Another data hazard we had to solve is LW-dependency. Which occur when we load a data from memory is needed in a following instruction that is in the execute stage. We solve it by stalling our first two stage registers and flushing our execute register.

The third hazard is a control hazard which occur when we are in a conditional branch instruction which is yet to decide where to jump. When the jump is not to the next instruction a hazard occur because we already started the following instructions but need to jump to a different location in the program. This hazard is solved by flushing our IF and ID stage registers.

The second problem we had is with the M-Unit. We needed to create the different operations exactly according to the RISC-V guidelines. The problem was to integrate this unit flawlessly into the existing RV32I Processor without compromising the other instructions that already work or the different modules that are affected.(קושי לשלב יחידת חישוב חדשה שמבצעת פעולות כבודת יותר מבלי לשבור את שאר המעבד שפועל כבר בצורה תקינה , כלומר לשמור על תאימות לRV32I  (

When getting into the final stages of the RISC-V Project we needed to design a driver for the 7 segments display as well as a frequency divider in order for our eye to see and process the data which is presented instead of it going in speeds that out eyes can comprehend.

The last challenge we had is in the simulation and after programing the FPGA. We needed a fast and reliable way to tell if the Processor we checked is working according to one of the 6 programs we ran on it. Therefore, we created a HASH code for each program the calculate according to live data if the program ran correctly or not. This method saved us a lot of time as we didn’t need to check the waveform and registers values throughout the program.

שקף 9

Adding support for peripherals to control the CPU and show more of its capabilities.

Create and connect a working assembler for the RISV-V Processor.

Optimize algorithms embedded in the CPU and saving on resources, essentially making the CPU even faster.

Implementing more extensions of RISC-V to extended the PROCESSOR capabilities.

שקף 10

We created a working RISC-V Soft-core that is fast and reliable executing all instructions correctly and according to the RISC-V guideline.

We also provided definite proof for superiority of Pipelined on Single cycle and Multi cycle when looking at it from an execution time stand point.

However, the pipelined took more resources  when implemented from the other two which correspond with what we opredicted.