RISC-V Instruction Set Architecture Extensions: A Survey

RISC-V is open-source and modular, making it ideal for tailored implementations. There are still gaps between what RISC-V currently offers and the needs of cutting-edge technologies.

The ISA (Instruction Set Architecture) is the core of every chip. Over the years, x86 and ARM have become dominant in the industry—but they are closed and costly. In contrast, RISC-V is open, free to use, and fosters innovation. It is already being widely adopted in regions like Europe and China, with projections pointing to tens of billions of cores. However, the current ISA still falls short of meeting the demands of areas such as cloud computing, AI, and graphics.

There are primary application domains of RISC-V , each domain requires different ISA extensions.

• Embedded Microprocessors (e.g., IoT, Smart Home, Wearables)
RISC-V is well-suited for embedded systems due to its modular architecture and support for customization. These systems often operate under tight power and area constraints and are expected to handle tasks such as signal processing, media encoding, encryption, and increasingly, on-device AI inference. To meet these requirements efficiently, there is a clear need for ISA extensions that are optimized for low-power operation, including lightweight floating-point units, vector processing capabilities tailored to AI workloads, and dedicated DSP instructions.

• Domain-Specific Processors
As general-purpose processors reach limitations in scalability and efficiency, domain-specific architectures have become a dominant trend. RISC-V’s openness and extensibility make it a strong candidate for designing such specialized processors. These include:

- GPUs, for high-throughput parallel graphics and AI tasks,

- DPUs, for data handling in cloud environments, and

- VCUs, for efficient video encoding and decoding.

Multiple projects have demonstrated the feasibility of domain-specific RISC-V implementations. Notably:

- Vortex (2020): a general-purpose GPU achieving 25.6 GFLOPS,

- Manticore (2021): a chiplet-based floating-point processor supporting up to 4096 cores,

- EdgeQ: a 5G communication SoC with over 50 custom ISA extensions.

In many cases, the standard RISC-V extensions do not fully address specialized computational needs. This drives the adoption of custom ISA enhancements, for instance, to accelerate graphics rendering or real-time signal processing in telecommunications.

• High-Performance Processors
RISC-V is gradually expanding into high-performance domains such as smartphones, laptops, cloud infrastructure, and supercomputing. While open-source processors like BOOMv3 and Xiangshan demonstrate promising performance, RISC-V still lacks some of the advanced features found in mature ISAs like x86 and ARM.
To close this gap, significant ISA extensions are required to support:

- compute-intensive workloads,

- real-time compilation (JIT),

- large-scale virtualization,

- and enhanced memory and security architectures.

Only by addressing these areas can RISC-V become a fully competitive option for high-end processing applications.

The official specification of the RISC-V architecture was defined by RISC-V International, the organization responsible for its standardization.

The RISC-V ISA is divided into two main components:

- Non-Privileged ISA – Covers user-level instructions and basic extensions.

- Privileged ISA – Encompasses machine and supervisor modes, including extensions for memory management and virtualization.

While RISC-V has made notable strides through various ISA extensions, it still lacks full support in certain advanced domains. These gaps present meaningful opportunities for developing domain-specific instructions—particularly in areas such as secure virtualization, cloud-native computing, and in-memory data processing—where performance and flexibility are critical.